|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 1 / 11 Document No : S 118 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| Safety Data Sheet According to Annex II to REACH - Regulation 2020/878 and to Annex II to UK REACH |  |  |
| SECTION 1. Identification of the substance/mixture and of the company/undertaking |  |  |
| - Product identifier Code: CRN MAM 156 Product name CRN SIL E-B - Relevant identified uses of the substance or mixture and uses advised against Intended use MICRO SILICONE SOFTENER - Details of the supplier of the safety data sheet Name CRN BOYA KIMYA SAN. TIC. LTD. STI. Full address ULUDAĞ ORGANİZE SANAYİ BÖLGESİ KALE MAH.KILIÇLAR CAD. NO:10 KESTEL District and Country 16450 BURSA TR Tel. +90 224 372 50 23 Fax +90 224 372 50 29 e-mail address of the competent person responsible for the Safety Data Sheet info@crnkimya.com.tr 1.4. Emergency telephone number For urgent inquiries refer to +90 224 372 50 23 |  |  |
| SECTION 2. Hazards identification |  |  |
| - Classification of the substance or mixture The product is classified as hazardous pursuant to the provisions set forth in (EC) Regulation 1272/2008 (CLP) (and subsequent amendments and supplements). The product thus requires a safety datasheet that complies with the provisions of (EU) Regulation 2020/878. Any additional information concerning the risks for health and/or the environment are given in sections 11 and 12 of this sheet. Hazard classification and indication: Serious eye damage, category 1 H318 Causes serious eye damage. Skin irritation, category 2 H315 Causes skin irritation. - Label elements Hazard labelling pursuant to EC Regulation 1272/2008 (CLP) and subsequent amendments and supplements. Hazard pictograms: Signal words: Danger Hazard statements: H318 Causes serious eye damage. H315 Causes skin irritation. Precautionary statements: P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P280 Wear protective gloves / eye protection / face protection. P310 Immediately call a POISON CENTER / doctor / P264 Wash thoroughly after handling. EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 2 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 2. Hazards identification ... / >> |  |  |
| Contains: ISOTRIDECYLALCOHOL 3 EO ISOTRIDECYLALCOHOL 8EO ACETIC ACID 2.3. Other hazards On the basis of available data, the product does not contain any PBT or vPvB in percentage ≥ than 0,1%. The product does not contain substances with endocrine disrupting properties in concentration ≥ 0.1%. |  |  |
| SECTION 3. Composition/information on ingredients |  |  |
| 3.2. Mixtures Contains: Identification x = Conc. % Classification (EC) 1272/2008 (CLP) WATER INDEX 65 ≤ x < 70 EC 231-791-2 CAS 7732-18-5 AMINOSILOXANE-SILICONE INDEX 20 ≤ x < 22.5 Skin Irrit. 2 H315 EC 600-354-1 CAS 102782-92-3 ISOTRIDECYLALCOHOL 3 EO INDEX 5 ≤ x < 6 Eye Dam. 1 H318 EC 934-965-2 CAS 69011-36-5 ISOTRIDECYLALCOHOL 8EO INDEX 2≤ x <3 Acute Tox. 4 H302, Eye Dam. 1 H318 EC 934-963-1 STA Oral: 500 mg/kg CAS 69011-36-5 2-(2-BUTOXYETHOXY)ETHANOL INDEX 603-096-00-8 2≤ x <3 Eye Irrit. 2 H319 EC 203-961-6 CAS 112-34-5 ACETIC ACID INDEX 607-002-00-6 0,5≤ x < 1.0 Flam. Liq. 3 H226, Skin Corr. 1A H314, Eye Dam. 1 H318, Classification note according to Annex VI to the CLP Regulation: B EC 200-580-7 Skin Corr. 1A H314: ≥ 90%, Skin Corr. 1B H314: ≥ 25%, Skin Irrit. 2 H315: ≥ 10%, Eye Dam. 1 H318: ≥ 25%, Eye Irrit. 2 H319: ≥ 10% CAS 64-19-7 The full wording of hazard (H) phrases is given in section 16 of the sheet. |  |  |
| SECTION 4. First aid measures |  |  |
| - Description of first aid measures EYES: Remove contact lenses, if present. Wash immediately with plenty of water for at least 30-60 minutes, opening the eyelids fully. Get medical advice/attention. SKIN: Remove contaminated clothing. Rinse skin with a shower immediately. Get medical advice/attention. INGESTION: Have the subject drink as much water as possible. Get medical advice/attention. Do not induce vomiting unless explicitly authorised by a doctor. INHALATION: Get medical advice/attention immediately. Remove victim to fresh air, away from the accident scene. If the subject stops breathing, administer artificial respiration. Take suitable precautions for rescue workers. - Most important symptoms and effects, both acute and delayed Specific information on symptoms and effects caused by the product are unknown. - Indication of any immediate medical attention and special treatment needed Information not available EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 3 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 5. Firefighting measures |  |  |
| - Extinguishing media SUITABLE EXTINGUISHING EQUIPMENT The extinguishing equipment should be of the conventional kind: carbon dioxide, foam, powder and water spray. UNSUITABLE EXTINGUISHING EQUIPMENT None in particular. - Special hazards arising from the substance or mixture HAZARDS CAUSED BY EXPOSURE IN THE EVENT OF FIRE Do not breathe combustion products. - Advice for firefighters GENERAL INFORMATION Use jets of water to cool the containers to prevent product decomposition and the development of substances potentially hazardous for health. Always wear full fire prevention gear. Collect extinguishing water to prevent it from draining into the sewer system. Dispose of contaminated water used for extinction and the remains of the fire according to applicable regulations. SPECIAL PROTECTIVE EQUIPMENT FOR FIRE-FIGHTERS Normal fire fighting clothing i.e. fire kit (BS EN 469), gloves (BS EN 659) and boots (HO specification A29 and A30) in combination with self-contained open circuit positive pressure compressed air breathing apparatus (BS EN 137). |  |  |
| SECTION 6. Accidental release measures |  |  |
| - Personal precautions, protective equipment and emergency procedures Block the leakage if there is no hazard. Wear suitable protective equipment (including personal protective equipment referred to under Section 8 of the safety data sheet) to prevent any contamination of skin, eyes and personal clothing. These indications apply for both processing staff and those involved in emergency procedures. - Environmental precautions The product must not penetrate into the sewer system or come into contact with surface water or ground water. - Methods and material for containment and cleaning up Collect the leaked product into a suitable container. Evaluate the compatibility of the container to be used, by checking section 10. Absorb the remainder with inert absorbent material. Make sure the leakage site is well aired. Contaminated material should be disposed of in compliance with the provisions set forth in point 13. - Reference to other sections Any information on personal protection and disposal is given in sections 8 and 13. |  |  |
| SECTION 7. Handling and storage |  |  |
| - Precautions for safe handling Keep away from heat, sparks and naked flames; do not smoke or use matches or lighters. Without adequate ventilation, vapours may accumulate at ground level and, if ignited, catch fire even at a distance, with the danger of backfire. Avoid bunching of electrostatic charges. Do not eat, drink or smoke during use. Remove any contaminated clothes and personal protective equipment before entering places in which people eat. Avoid leakage of the product into the environment. - Conditions for safe storage, including any incompatibilities Store only in the original container. Store in a cool and well ventilated place, keep far away from sources of heat, naked flames and sparks and other sources of ignition. Keep containers away from any incompatible materials, see section 10 for details. - Specific end use(s) Information not available EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 4 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 8. Exposure controls/personal protection |  |  |
| - Control parameters Regulatory References: TUR Türkiye Kimyasal Maddelerle Çalışmalarda Sağlık ve Güvenlik Önlemleri Hakkında Yönetmelik 12.08.2013 / 28733 GBR United Kingdom EH40/2005 Workplace exposure limits (Fourth Edition 2020) EU OEL EU Directive (EU) 2022/431; Directive (EU) 2019/1831; Directive (EU) 2019/130; Directive (EU) 2019/983; Directive (EU) 2017/2398; Directive (EU) 2017/164; Directive 2009/161/EU; Directive 2006/15/EC; Directive 2004/37/EC; Directive 2000/39/EC; Directive 98/24/EC; Directive 91/322/EEC. TLV-ACGIH ACGIH 2021 ESD TUR 67.5 10 101.2 15 WEL GBR 67.5 10 101.2 15 OEL EU 67.5 10 101.2 15 TLV-ACGIH 66 10 INHAL ESD TUR 25 10 WEL GBR 25 10 50 20 OEL EU 25 10 50 20 TLV-ACGIH 25 10 37 15 Legend: (C) = CEILING ; INHAL = Inhalable Fraction ; RESP = Respirable Fraction ; THORA = Thoracic Fraction. - Exposure controls As the use of adequate technical equipment must always take priority over personal protective equipment, make sure that the workplace is well aired through effective local aspiration. When choosing personal protective equipment, ask your chemical substance supplier for advice. Personal protective equipment must be CE marked, showing that it complies with applicable standards. Provide an emergency shower with face and eye wash station. HAND PROTECTION Protect hands with category III work gloves (see standard EN 374). The following should be considered when choosing work glove material: compatibility, degradation, failure time and permeability. The work gloves' resistance to chemical agents should be checked before use, as it can be unpredictable. The gloves' wear time depends on the duration and type of use. SKIN PROTECTION Wear category II professional long-sleeved overalls and safety footwear (see Regulation 2016/425 and standard EN ISO 20344). Wash body with soap and water after removing protective clothing. EYE PROTECTION Wear airtight protective goggles (see standard EN 166). RESPIRATORY PROTECTION If the threshold value (e.g. TLV-TWA) is exceeded for the substance or one of the substances present in the product, use a mask with a type A filter whose class (1, 2 or 3) must be chosen according to the limit of use concentration. (see standard EN 14387). In the presence of gases or vapours of various kinds and/or gases or vapours containing particulate (aerosol sprays, fumes, mists, etc.) combined filters are required. Respiratory protection devices must be used if the technical measures adopted are not suitable for restricting the worker's exposure to the threshold values considered. The protection provided by masks is in any case limited. If the substance considered is odourless or its olfactory threshold is higher than the corresponding TLV-TWA and in the case of an emergency, wear open-circuit compressed air breathing apparatus (in compliance with standard EN 137) or external air-intake breathing apparatus (in compliance with standard EN 138). For a correct choice of respiratory protection device, see standard EN 529. ENVIRONMENTAL EXPOSURE CONTROLS The emissions generated by manufacturing processes, including those generated by ventilation equipment, should be checked to ensure compliance with environmental standards. EPY 11.3.0 - SDS 1004.14 |  |  |

| 2-(2-BUTOXYETHOXY)ETHANOL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Threshold Limit Value |  |  |  |  |  |
| Type | Country | TWA/8h |  | STEL/15min | Remarks / Observations |
|  |  | mg/m3 | ppm | mg/m3 ppm |  |

| ACETIC ACID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Threshold Limit Value |  |  |  |  |  |
| Type | Country | TWA/8h |  | STEL/15min | Remarks / Observations |
|  |  | mg/m3 | ppm | mg/m3 ppm |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 5 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 9. Physical and chemical properties |  |  |
| - Information on basic physical and chemical properties Properties Value Information Appearance Clear, viscose liquid Colour colourless Odour pungent Melting point / freezing point not available Initial boiling point not available Flammability not available Lower explosive limit not available Upper explosive limit not available Flash point not available Auto-ignition temperature not available Decomposition temperature not available pH (10%) 4,0- 6,0 Kinematic viscosity not available Solubility not available Partition coefficient: n-octanol/water not available Vapour pressure not available Density and/or relative density 0,98-0,99 g/cm3 Relative vapour density not available Particle characteristics not applicable - Other information - Information with regard to physical hazard classes Information not available - Other safety characteristics Information not available |  |  |
| SECTION 10. Stability and reactivity |  |  |
| - Reactivity There are no particular risks of reaction with other substances in normal conditions of use. - Chemical stability The product is stable in normal conditions of use and storage. - Possibility of hazardous reactions The vapours may also form explosive mixtures with the air. 2-(2-BUTOXYETHOXY)ETHANOL May react with: oxidising substances.May form peroxides with: oxygen.Develops hydrogen on contact with: aluminium.May form explosive mixtures with: air. ACETIC ACID Risk of explosion on contact with: chromium (VI) oxide,potassium permanganate,sodium peroxide,perchloric acid,phosphorus chloride,hydrogen peroxide.May react dangerously with: alcohols,bromine pentafluoride,chlorosulphuric acid,dichromate-sulphuric acid,ethane diamine,ethylene glycol,potassiun hydroxide,strong bases,sodium hydroxide,strong oxidising agents,nitric acid,ammonium nitrate,potassium tert-butoxide,oleum.Forms explosive mixtures with: air. - Conditions to avoid Avoid overheating. Avoid bunching of electrostatic charges. Avoid all sources of ignition. 2-(2-BUTOXYETHOXY)ETHANOL Avoid exposure to: air. ACETIC ACID Avoid exposure to: sources of heat,naked flames. - Incompatible materials 2-(2-BUTOXYETHOXY)ETHANOL EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 6 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 10. Stability and reactivity ... / >> |  |  |
| Incompatible with: oxidising substances,strong acids,alkaline metals. ACETIC ACID Incompatible with: carbonates,hydroxides,phosphates,oxidising substances,bases. 10.6. Hazardous decomposition products In the event of thermal decomposition or fire, gases and vapours that are potentially dangerous to health may be released. 2-(2-BUTOXYETHOXY)ETHANOL May develop: hydrogen. |  |  |
| SECTION 11. Toxicological information |  |  |
| In the absence of experimental data for the product itself, health hazards are evaluated according to the properties of the substances it contains, using the criteria specified in the applicable regulation for classification. It is therefore necessary to take into account the concentration of the individual hazardous substances indicated in section 3, to evaluate the toxicological effects of exposure to the product. 11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008 Metabolism, toxicokinetics, mechanism of action and other information Information not available Information on likely routes of exposure 2-(2-BUTOXYETHOXY)ETHANOL WORKERS: inhalation; contact with the skin. Delayed and immediate effects as well as chronic effects from short and long-term exposure 2-(2-BUTOXYETHOXY)ETHANOL May be absorbed by inhalation, ingestion and skin contact; is irritating for the skin and especially for the eyes. May cause damage to the spleen. At room temperature the danger of inhalation is unlikely, due to the low vapour pressure of the substance. Interactive effects Information not available ACUTE TOXICITY ATE (Inhalation) of the mixture: Not classified (no significant component) ATE (Oral) of the mixture: >2000 mg/kg ATE (Dermal) of the mixture: Not classified (no significant component) 2-(2-BUTOXYETHOXY)ETHANOL LD50 (Dermal): 2700 mg/kg Rabbit LD50 (Oral): 3384 mg/kg Rat ACETIC ACID LD50 (Dermal): 1060 mg/kg Rabbit LD50 (Oral): 3310 mg/kg Rat LC50 (Inhalation vapours): 11.4 mg/l/4h Rat ISOTRIDECYLALCOHOL 8EO LD50 (Dermal): > 2000 mg/kg LD50 (Oral): > 2000 mg/kg STA (Oral): 500 mg/kg estimate from table 3.1.2 of Annex I of the CLP (figure used for calculation of the acute toxicity estimate of the mixture) SKIN CORROSION / IRRITATION Causes skin irritation SERIOUS EYE DAMAGE / IRRITATION Causes serious eye damage RESPIRATORY OR SKIN SENSITISATION EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 7 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 11. Toxicological information ... / >> |  |  |
| Does not meet the classification criteria for this hazard class GERM CELL MUTAGENICITY Does not meet the classification criteria for this hazard class CARCINOGENICITY Does not meet the classification criteria for this hazard class REPRODUCTIVE TOXICITY Does not meet the classification criteria for this hazard class STOT - SINGLE EXPOSURE Does not meet the classification criteria for this hazard class STOT - REPEATED EXPOSURE Does not meet the classification criteria for this hazard class ASPIRATION HAZARD Does not meet the classification criteria for this hazard class 11.2. Information on other hazards Based on the available data, the product does not contain substances listed in the main European lists of potential or suspected endocrine disruptors with human health effects under evaluation. |  |  |
| SECTION 12. Ecological information |  |  |
| Use this product according to good working practices. Avoid littering. Inform the competent authorities, should the product reach waterways or contaminate soil or vegetation. - Toxicity 2-(2-BUTOXYETHOXY)ETHANOL LC50 - for Fish 1300 mg/l/96h EC50 - for Crustacea > 100 mg/l/48h ISOTRIDECYLALCOHOL 8EO LC50 - for Fish > 1 mg/l/96h EC50 - for Crustacea > 1 mg/l/48h EC50 - for Algae / Aquatic Plants > 1 mg/l/72h - Persistence and degradability 2-(2-BUTOXYETHOXY)ETHANOL Solubility in water 1000 - 10000 mg/l Rapidly degradable ACETIC ACID Solubility in water > 10000 mg/l Rapidly degradable - Bioaccumulative potential 2-(2-BUTOXYETHOXY)ETHANOL Partition coefficient: n-octanol/water 1 ACETIC ACID Partition coefficient: n-octanol/water -0.17 - Mobility in soil EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 8 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 12. Ecological information ... / >> |  |  |
| ACETIC ACID Partition coefficient: soil/water 1.153 - Results of PBT and vPvB assessment On the basis of available data, the product does not contain any PBT or vPvB in percentage ≥ than 0,1%. - Endocrine disrupting properties Based on the available data, the product does not contain substances listed in the main European lists of potential or suspected endocrine disruptors with environmental effects under evaluation. - Other adverse effects Information not available |  |  |
| SECTION 13. Disposal considerations |  |  |
| 13.1. Waste treatment methods Reuse, when possible. Product residues should be considered special hazardous waste. The hazard level of waste containing this product should be evaluated according to applicable regulations. Disposal must be performed through an authorised waste management firm, in compliance with national and local regulations. CONTAMINATED PACKAGING Contaminated packaging must be recovered or disposed of in compliance with national waste management regulations. |  |  |
| SECTION 14. Transport information |  |  |
| The product is not dangerous under current provisions of the Code of International Carriage of Dangerous Goods by Road (ADR) and by Rail (RID), of the International Maritime Dangerous Goods Code (IMDG), and of the International Air Transport Association (IATA) regulations. - UN number or ID number not applicable - UN proper shipping name not applicable - Transport hazard class(es) not applicable - Packing group not applicable - Environmental hazards not applicable - Special precautions for user not applicable - Maritime transport in bulk according to IMO instruments Information not relevant EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 9 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 15. Regulatory information |  |  |
| - Safety, health and environmental regulations/legislation specific for the substance or mixture Seveso Category - Directive 2012/18/EU: None Restrictions relating to the product or contained substances pursuant to Annex XVII to EC Regulation 1907/2006 Product Point 3 - 40 Contained substance Point 75 Point 55 2-(2-BUTOXYETHOXY)ETHANOL Regulation (EU) 2019/1148 - on the marketing and use of explosives precursors not applicable Substances in Candidate List (Art. 59 REACH) On the basis of available data, the product does not contain any SVHC in percentage ≥ than 0,1%. Substances subject to authorisation (Annex XIV REACH) None Substances subject to exportation reporting pursuant to Regulation (EU) 649/2012: None Substances subject to the Rotterdam Convention: None Substances subject to the Stockholm Convention: None Healthcare controls Workers exposed to this chemical agent must not undergo health checks, provided that available risk-assessment data prove that the risks related to the workers' health and safety are modest and that the 98/24/EC directive is respected. - Chemical safety assessment A chemical safety assessment has not been performed for the preparation/for the substances indicated in section 3. |  |  |
| SECTION 16. Other information |  |  |
| Text of hazard (H) indications mentioned in section 2-3 of the sheet: Flam. Liq. 3 Flammable liquid, category 3 Acute Tox. 4 Acute toxicity, category 4 Skin Corr. 1A Skin corrosion, category 1A Eye Dam. 1 Serious eye damage, category 1 Eye Irrit. 2 Eye irritation, category 2 Skin Irrit. 2 Skin irritation, category 2 H226 Flammable liquid and vapour. H302 Harmful if swallowed. H314 Causes severe skin burns and eye damage. H318 Causes serious eye damage. H319 Causes serious eye irritation. H315 Causes skin irritation. LEGEND: - ADR: European Agreement concerning the carriage of Dangerous goods by Road - ATE: Acute Toxicity Estimate - CAS: Chemical Abstract Service Number - CE50: Effective concentration (required to induce a 50% effect) - CE: Identifier in ESIS (European archive of existing substances) - CLP: Regulation (EC) 1272/2008 - DNEL: Derived No Effect Level - EmS: Emergency Schedule - GHS: Globally Harmonized System of classification and labeling of chemicals EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 10 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| SECTION 16. Other information ... / >> |  |  |
| - IATA DGR: International Air Transport Association Dangerous Goods Regulation - IC50: Immobilization Concentration 50% - IMDG: International Maritime Code for dangerous goods - IMO: International Maritime Organization - INDEX: Identifier in Annex VI of CLP - LC50: Lethal Concentration 50% - LD50: Lethal dose 50% - OEL: Occupational Exposure Level - PBT: Persistent bioaccumulative and toxic as REACH Regulation - PEC: Predicted environmental Concentration - PEL: Predicted exposure level - PNEC: Predicted no effect concentration - REACH: Regulation (EC) 1907/2006 - RID: Regulation concerning the international transport of dangerous goods by train - TLV: Threshold Limit Value - TLV CEILING: Concentration that should not be exceeded during any time of occupational exposure. - TWA: Time-weighted average exposure limit - TWA STEL: Short-term exposure limit - VOC: Volatile organic Compounds - vPvB: Very Persistent and very Bioaccumulative as for REACH Regulation - WGK: Water hazard classes (German). GENERAL BIBLIOGRAPHY - Regulation (EC) 1907/2006 (REACH) of the European Parliament - Regulation (EC) 1272/2008 (CLP) of the European Parliament - Regulation (EU) 2020/878 (II Annex of REACH Regulation) - Regulation (EC) 790/2009 (I Atp. CLP) of the European Parliament - Regulation (EU) 286/2011 (II Atp. CLP) of the European Parliament - Regulation (EU) 618/2012 (III Atp. CLP) of the European Parliament - Regulation (EU) 487/2013 (IV Atp. CLP) of the European Parliament - Regulation (EU) 944/2013 (V Atp. CLP) of the European Parliament - Regulation (EU) 605/2014 (VI Atp. CLP) of the European Parliament - Regulation (EU) 2015/1221 (VII Atp. CLP) of the European Parliament - Regulation (EU) 2016/918 (VIII Atp. CLP) of the European Parliament - Regulation (EU) 2016/1179 (IX Atp. CLP) - Regulation (EU) 2017/776 (X Atp. CLP) - Regulation (EU) 2018/669 (XI Atp. CLP) - Regulation (EU) 2019/521 (XII Atp. CLP) - Delegated Regulation (UE) 2018/1480 (XIII Atp. CLP) 17. Regulation (EU) 2019/1148 - Delegated Regulation (UE) 2020/217 (XIV Atp. CLP) - Delegated Regulation (UE) 2020/1182 (XV Atp. CLP) - Delegated Regulation (UE) 2021/643 (XVI Atp. CLP) - Delegated Regulation (UE) 2021/849 (XVII Atp. CLP) - Delegated Regulation (UE) 2022/692 (XVIII Atp. CLP) - The Merck Index. - 10th Edition - Handling Chemical Safety - INRS - Fiche Toxicologique (toxicological sheet) - Patty - Industrial Hygiene and Toxicology - N.I. Sax - Dangerous properties of Industrial Materials-7, 1989 Edition - IFA GESTIS website - ECHA website - Database of SDS models for chemicals - Ministry of Health and ISS (Istituto Superiore di Sanità) - Italy Note for users: The information contained in the present sheet are based on our own knowledge on the date of the last version. Users must verify the suitability and thoroughness of provided information according to each specific use of the product. This document must not be regarded as a guarantee on any specific product property. The use of this product is not subject to our direct control; therefore, users must, under their own responsibility, comply with the current health and safety laws and regulations. The producer is relieved from any liability arising from improper uses. Provide appointed staff with adequate training on how to use chemical products. EPY 11.3.0 - SDS 1004.14 |  |  |

|  | CRN BOYA KIMYA SAN. TIC. LTD. STI. | Revision nr.1 EN Dated 26/01/2023 First compilation Printed on 26/01/2023 Page n. 11 / 11 Document No : S 115 |
| --- | --- | --- |
|  | CRN SIL E-B |  |
| CALCULATION METHODS FOR CLASSIFICATION Chemical and physical hazards: Product classification derives from criteria established by the CLP Regulation, Annex I, Part 2. The data for evaluation of chemical-physical properties are reported in section 9. Health hazards: Product classification is based on calculation methods as per Annex I of CLP, Part 3, unless determined otherwise in Section 11. Environmental hazards: Product classification is based on calculation methods as per Annex I of CLP, Part 4, unless determined otherwise in Section 12. EPY 11.3.0 - SDS 1004.14 |  |  |